BT+APRS board FW: BHT_BOARD_V1.03_20220301.spi, improve the analog APRS decode

For D878UV:

1. Press and hold both top key and side PF2 key, and power the radio ON, the radio display “UPDATE MODE FOR LinkBoard”

2. Open the Programming Software (CPS) and go to TOOLS > Firmware Update

3.Select the Open File button and select the appropriate BHT_BOARD_V1.03_20220301.spi file in the FW Update folder

4. Check and Set the Com Port that is correct for your PC connected to the radio.

5. Select the Write button to write the new FW to the radio.

For D578UV:

- Press and hold both PUSH key and P6 key, and power the radio ON, the radio display “UPDATE MODE FOR LinkBoard”

- Open the Programming Software (CPS) and go to TOOLS > Firmware Update

- Select the Open File button and select the appropriate BHT_BOARD_V1.03_20220301.spi file in the FW Update folder

- Check and Set the Com Port that is correct for your PC connected to the radio.

- Select the Write button to write the new FW to the radio.